# Transport Layer Security & IEEE 802.11 Wireless Security
Complete Exam Handbook

These notes are compiled from the complete unit and rewritten into easy-to-learn exam notes. They are intended for quick revision while still containing enough depth to score well in descriptive answers.

## 1. Web Security Considerations

### Why Web Security?

Web browsers and servers are easy to use but their underlying software is complex and may contain vulnerabilities.

A compromised web server can become an entry point into the organization's internal network.

Many users are not security-aware, making phishing and malware attacks easier.

### Passive vs Active Attacks

Passive Attacks: Eavesdropping, traffic analysis, unauthorized reading of data. They observe but do not modify data.

Active Attacks: Impersonation, message modification, website tampering and replay attacks. They alter or disrupt communication.

### Major Threats

Integrity → Unauthorized modification of data. Countermeasure: Cryptographic checksums/MAC.

Confidentiality → Unauthorized disclosure of data. Countermeasure: Encryption.

Authentication → Impersonation and identity forgery. Countermeasure: Digital certificates and cryptography.

Denial of Service → Prevents legitimate users from accessing services.

### Web Traffic Security Approaches

IPSec protects all IP traffic at the network layer.

SSL/TLS secures communication above TCP.

Application-specific security protects individual applications.

## 2. Secure Socket Layer (SSL)

SSL provides Confidentiality, Integrity and Authentication over TCP.

SSL architecture consists of SSL Record Protocol and three higher-layer protocols: Handshake, Change Cipher Spec and Alert.

### SSL Record Protocol

Services: Confidentiality (Symmetric Encryption) and Integrity (MAC).

Operation: Fragmentation → Compression → MAC → Encryption → SSL Header.

Header fields: Content Type, Version, Length.

### SSL Handshake

Purpose: Authenticate peers, negotiate cipher suite, exchange keys.

Phases: Establish Security Capabilities → Server Authentication & Key Exchange → Client Authentication & Key Exchange → Finish.

### Change Cipher Spec

One-byte protocol that activates the newly negotiated cipher suite and keys.

### Alert Protocol

Reports warnings and fatal errors.

Fatal alerts terminate the SSL connection immediately.

### SSL Session vs Connection

Session: Long-lived association created by the Handshake Protocol.

Connection: Temporary secure communication link using one session.

## 3. Transport Layer Security (TLS)

TLS is the successor to SSL and is the standard protocol for Internet security.

Uses public-key cryptography for authentication and key exchange.

Uses symmetric encryption (AES etc.) for fast data encryption.

Uses hash functions for integrity verification.

## 4. HTTPS

HTTPS = HTTP + TLS.

Uses Port 443 instead of Port 80.

Encrypts URLs, page contents, cookies, form data and HTTP headers.

Working: Connection → TLS Handshake → Secure HTTP Requests → close_notify → TCP Close.

## 5. Wireless Security

Wireless networks are more vulnerable because communication is broadcast.

Major risks: Channel attacks, Mobility, Limited Resources and Accessibility.

Common protections include authentication, encryption, secure updates and VPNs.

## 6. IEEE 802.11 WLAN

### Architecture

LLC: Flow and error control.

MAC: Framing, addressing and medium access.

Physical Layer: Signal transmission.

### Terminology

STA: Wireless station/device.

AP: Access Point.

BSS: One AP and its associated stations.

DS: Distribution System connecting BSSs.

ESS: Multiple BSSs connected by a DS.

### MSDU vs MPDU

MSDU = Actual user data.

MPDU = MAC Header + MSDU + CRC.

### Data Delivery Services

MSDU Delivery: Delivers user data.

Distribution: Transfers MPDUs between BSSs through the Distribution System.

Integration: Transfers data between IEEE 802.11 WLAN and wired IEEE 802.x LAN.

### Association Services

Association: Initial connection with an AP.

Reassociation: Move from one AP to another.

Disassociation: Terminates an association.

### Mobility Types

No Transition: Station remains in one BSS.

BSS Transition: Moves between BSSs within one ESS.

ESS Transition: Moves to another ESS.

### Security Evolution

WEP → WPA → RSN (IEEE 802.11i) → WPA2

## 7. IEEE 802.11i (RSN)

RSN (Robust Security Network) is the final IEEE 802.11i security framework.

Provides Access Control, Authentication, Key Management, Confidentiality, Integrity and Replay Protection.

### Main Elements

IEEE 802.1X: Port-based Access Control.

EAP: Authentication and Key Generation.

TKIP: RC4-based security for WPA.

CCMP: AES-based security for WPA2.

### Phases of Operation

Entities: STA, AP, AS (Authentication Server), End Station.

1. Discovery

2. Authentication

3. Key Management

4. Protected Data Transfer

5. Connection Termination

Memory Trick: D-A-K-P-T

## Quick Revision

Passive = Observe | Active = Modify

SSL Record: Fragment → Compress → MAC → Encrypt → Header

HTTPS = HTTP + TLS (Port 443)

MSDU = User Data | MPDU = Header + MSDU + CRC

Distribution = WLAN ↔ WLAN

Integration = WLAN ↔ Wired LAN

WEP → WPA → RSN → WPA2

IEEE 802.11i: 802.1X + EAP + CCMP(AES)